Quotation and Specification Sheet for Fence Semi Trailer

| Quotation: Description of goods; Quantity; Unit Price (FOB Tianjin); Unit Price (CIF ); Shipment. 40FT Fence semi trailer; 1 unit; 11000USD/unit; 40FT/Bulk cargo ship. Terms & Conditions: |
| --- |
| Spec.: | Please see the enclosed specification |
| Delivery: | 30 days after down payment received |
| Payment: | Down payment 30% by T/T within 5 days after order confirmation Balance 70% by T/T before EXW. |
| Warranty: | 12 Months |

| Specification: |  |  |
| --- | --- | --- |
| Type: |  | 40FT Fence semi trailer |
| Model: |  | SH9400CCYE |
| Dimension: |  |  |
|  | Length: | 12,400mm |
|  | Width: | 2,500mm |
|  | Drop Side Height: | 1800mm |
|  | Kingpin location: | 1,045mm from the front face of the front bolster |
|  | Axle spacer: | 7,200mm+1,310mm+1,310mm |
| (The above dimension will be adjusted according to the tractor specification.) |  |  |
| Weight: |  |  |
|  | Payload: | 40,000kg |
|  | Tare weight: | Approx. 8,000kg |
|  | G.V.W.R.: | Approx. 48,000kg |
| Frame & Structure: |  |  |
|  | Main beam: | Material Q345B. Fabricated “H” beam height 500mm, Top flange thickness 14mm, Middle web thickness 8mm, Bottom flange thickness 16mm |
|  | Side beam: | 16# channel steel |
|  | Platform: | 3mm thick chequer plate with supporting channel |
|  | Cross beam: | 10# channel steel |
|  | Kingpin: | 2” Kingpin, bolted in type |
|  | Welding standard: | All welding are to be done by qualified welder per applicable national standard |
|  | Painting: | Shot blast at Standard SA 2.5 prior to application of primer, water paint top coat. Total DFT not less than 80um |
| Running gear: |  |  |
|  | Suspension: | Standard mechanical suspension, 10 pcs leaf spring 13mmx90mm |
|  | Axle: | Open brand axles, 3pcs*13 ton capacity |
|  | Brake system: | Dual line pneumatic brake, with hose and connector |
|  | Brake chamber: | 4 units T30/30 spring brake chambers |
|  | Air tank: | 40L, 2 pcs, China |
|  | Emergency relay valve: | Wabco relay valve |
|  | Tire: | Linglong brand, 12R22.5, 12 pcs |
|  | Rim: | 9.00-22.5 steel disc wheel,10 holes ISO, 12 pcs |
|  | Landing gear: | 28T two speed telescopic manual landing gear |

| Electrical System: |  |  |
| --- | --- | --- |
|  | Voltage: | 24 Volt |
|  | Receptacle: | 7 way socket standard, China |
|  | Front marker lamp: | White |
|  | Side marker lamp: | White & Red |
|  | Rear lamp: | Red |
|  | Turn signal lamp: | Amber |
|  | Reflector: | Per customer requirement |
| Accessories: |  |  |
|  | Mud guard: | Steel mud guard |
|  | Spare wheel carriers: | 1 sets |
|  | Tool box: | 1 pc |
| Note: All the details not mentioned above should comply with Chinese regulations. If any component or structure changed, the price shall be modified accordingly. |  |  |

Our Factories